# SFS LoRa Sensor Node Firmware — Open Source Software Documentation

## 1. Software Identification

Software Title: SFS LoRa Sensor Node Firmware
Short Description: Open-source firmware for the SFS LoRa sensor node. Reads distance (ToF), temperature, and humidity sensors, applies filtering, and transmits encoded readings to the concentrator node over LoRa at 10-second intervals.
Keywords/Tags: LoRa, sensor node, IoT, livestock, feed monitoring, STM32, embedded C, ultrasonic, ToF, OpenAgri
Author(s)/Maintainer(s): Inokron FWSW Team
Organization: Inokron d.o.o.
Contact Email: info@inokron.com
Repository URL: https://github.com/inokron-doo/OA-Sensor
Version: v1.0.0
Programming Language(s): C (STM32 HAL, C99)
Operating System(s): Bare metal (no OS) — STM32F051K8T7 Cortex-M0

## 2. Purpose and Scope

The sensor node firmware acquires feed-level distance measurements and in-barn climate data (temperature, humidity) and transmits them at regular intervals to the concentrator node over LoRa. Up to four DFRobot SEN0648 ToF distance sensors can be connected per node via an onboard 4-port UART multiplexer. One Sensirion SHT85 climate sensor is supported per node via I²C.

This firmware is the field-deployed counterpart to the SFS LoRa Concentrator Node Firmware.

## 3. Features and Functionality

Up to 4 DFRobot SEN0648 ToF distance sensors via 4-port UART multiplexer;

Automatic sensor port detection on startup; periodic re-probe every 3 minutes;

95th-percentile filter on distance samples (~100 samples per 10 s window) to reject animal-head interference;

Sensirion SHT85 temperature and humidity sensing via I²C;

Round-robin LoRa transmission: temperature → humidity → distance sensors, one slot per 10 s;

LED status: WRK heartbeat (1 Hz), COMM activity, R_ERROR fault indicator;

Ultrasonic sensor decode pipeline (AJ-SR04M / A01NYUB) implemented but not active;

## 4. System Architecture

Interrupt-driven UART reception from up to four multiplexed ToF sensors feeds per-sensor sample buffers; a 10-second timer tick drives slot-based LoRa transmission via a second UART. Climate data is acquired over I²C on each transmission cycle. No RTOS — all logic runs in a single main loop with a hardware IWDG for fault recovery.

## 5. Installation and Setup

System requirements: STM32CubeIDE, ST-Link V2 programmer;

Sensors: One or more DFRobot SEN0648 connected to SENSOR ports 1–4;

Build: Import project into STM32CubeIDE, set #define SENSOR_DEVICE_ID <1–31> in main.c, build Release configuration, flash via ST-Link

## 6. Usage Instructions

Power node via PCB terminal block. On startup, sensor ports are probed and the active set logged over USART2 (115200 baud). Node then transmits autonomously; no user interaction required. Monitor USART2 for debug output or connect USART1 output to the concentrator node. WRK LED heartbeat (1 Hz) confirms normal operation; R_ERROR indicates sensor fault.

## 7. Input/Output Specifications

Input: SEN0648 ToF frames (16-byte, USART2, 115200 baud); SHT85 6-byte I²C response (address 0x44, command 0x2400)

Output: 6-byte LoRa broadcast packet (USART1, 9600 baud)

## 8. API Documentation (If Applicable)

Not applicable

## 9. Software Dependencies

STM32F0xx HAL, CMSIS (ARM Cortex-M0) — both bundled with STM32CubeIDE. No external libraries.

## 10. Testing and Validation (optional)

No automated test harness. Validated through field deployment at Novak farm (Gorenjska, Slovenia)

## 11. Performance and Scalability (optional)

~100 distance samples per sensor per 10 s window. Up to 4 sensors per node; up to 32 nodes per concentrator (5-bit device ID).

## 12. Security Considerations

LoRa channel is unencrypted broadcast — intentional for farm environment. Physical USART2 debug access should not be exposed in production enclosures.

## 13. Licensing and Usage Rights

Licence: GNU General Public License v3.0 (GPL-3.0). Derivative works must be released under the same licence with source code made available.

Attribution: Retain copyright notice and repository reference in all copies or derivative works.

Commercial use: Permitted subject to GPL-3.0 terms.

## 14. Contribution Guidelines

Fork repository, create feature branch, submit merge request. Keep user code within USER CODE blocks to maintain STM32CubeIDE compatibility. Test on hardware before submitting. Report issues via repository issue tracker.

## 15. Versioning and Changelog (optional)

Versioning scheme: Semantic versioning (MAJOR.MINOR.PATCH)
Changelog:

- v1.0.0 – 2026-06-17 – Initial release

## 16. Roadmap and Future Work

- Runtime node ID and configuration (transmission interval, sensor parameters) delivered via LoRa downlink, eliminating the need for per-node firmware builds.

- Extended sensor support and field evaluation of alternative distance measurement technologies, including structured-light grid sensors that could provide area-based trough coverage rather than single-point depth measurement.

## 17. Known Limitations

Node ID is compile-time only, each node requires a separate firmware build.

Single-point distance measurement can be noisy at low feed levels. Animal feeding behaviour displaces feed into mounds and depressions; a single sensor reading captures one point on that surface, increasing variance even after 4 sensor aggregation and filtering.

## 18. Citation and Acknowledgements (optional)

Citation: Inokron d.o.o. (2026). SFS LoRa Sensor Node Firmware

Funding: Developed under the OpenAgri project, Horizon Europe programme, SIP 13

## 19. Contact and Support

Maintainer name: Inokron FWSW Team
Institution: Inokron d.o.o.
Support email/Raising issues: info@inokron.com